INSTRUCTIONS FOR CARE AND CONTINUED AIRWORTHINESS

Ashby Glare Shield Products

Document IC-001

INSTRUCTIONS FOR CARE AND CONTINUED AIRWORTHINESS

Ashby Glare Shield Products

This document IC-001 is applicable to the following glare shield models:

PN 172L

PN 28

PN 6

PN 182

PN 180

INSTRUCTIONS FOR CARE AND CONTINUED AIRWORTHINESS

| RECORD OF REVISIONS IC-001 |  |  |  |
| --- | --- | --- | --- |
| REV | SHEET | DESCRIPTION | DATE APPROVED |
| A | ALL | INITIAL RELEASE | 2/1/02 |
| B | ALL | Added: Airworthiness Limitations | 9/1/04 |
| C | ALL | Reformat to comply with 14 CFR 23 Appendix G23.4 | 4/21/2005 |

Table of Contents

Page

1. Introduction  					4

2. Instructions for Care				4

3. Inspection 					4

4. Removal					4

5. Weight and Balance				4

6. Airworthiness Limitations			5

7. Program for Dissemination of Changes in 		5

the Instructions for Continued Airworthiness

8. Contact Information				5

9. Other information related to Appendix G, Part 23   6 	  requirements

1. Introduction

The Ashby Glare Shield is designed for many years of use with a minimal amount of care.  The glare shield is manufactured using a flame-resistant material that has been tested to 14 CFR 23.853(a). amdt 25-49.  This document is intended to provide instruction on how to maintain the glare shield to achieve many years of safe and reliable service.

2. Instructions for Care

To preserve the fire retardant qualities of your glare shield, and to prevent warping, de-lamination or other deformation, the following procedure should be followed:

a. Use only mild soap and water to clean the glareshield.  Always use a soft non-abrasive cloth.  Never use abrasives.   Never use petroleum-based solvents or other harsh chemicals or treatments.  It is recommended that the glareshield be cleaned at each annual inspection, or equivalent.

b.   Never paint the glareshield except with materials approved by the manufacturer, or apply decals or coverings to the glareshield.  These could counter the self-extinguishing characteristics of the materials.

c.   The glare shield is not designed as a structural member of the airframe and is therefore not to be used as a primary attachment point for instruments.  It is acceptable to mount instruments on top of the glare shield provided the method of attachment is to an approved underlying structure.

d. The glare shield must be mounted securely to the airframe without noticeable vibration or flexing during aircraft operations.

# 3. Inspection

1.  The glare shield should be inspected at each annual inspection for mounting security.

# 4. Removal

2. The glare shield may be removed by first removing the attachment screws, then removing the glare shield from the panel.

# 5. Weight and Balance

The glare shield does not represent a significant weight factor in the affected aircraft and no recalculation of Weight and Balance is required.

# 6. Airworthiness Limitations

The Airworthiness Limitations section is FAA approved and specifies maintenance required under Secs. 43.16 and 91.403 of the Federal Aviation Regulations unless an alternative program has been FAA Approved.

When installed according to Installation Instructions (IN001) and maintained according to the provisions of this document (IC001), there are no additional airworthiness limitations imposed on the aircraft other than those included in the original Type Certificate, or otherwise existing prior to installation of the glare shield.

7. Program for Dissemination of Changes in the Instructions for Continue Airworthiness

The manufacturer maintains a list of addresses for original purchasers of the Ashby Glare Shield.  In the event of a change in the Instructions for Continued Airworthiness, a revised version will be mailed based upon this list.

8. Contact Information

Questions related to the Ashby Glare Shield may be directed to:

Dennis Ashby

P.O. Box 1584

Upland, CA  91785-1584

909 466 3918

9. Other information related to Appendix G, Part 23 requirements

Basic Control and Operating Information			N/A

Servicing information regarding Pressures and Capacities	N/A

Location of Access Panels				N/A

Servicing information regarding lubrication			N/A

Equipment required for servicing				N/A

Tow instructions					N/A

Mooring information					N/A

Jacking information					N/A

Leveling information					N/A

Scheduling information for the airplane’s engine(s)		N/A

Scheduling information for the airplane’s APU		N/A

Scheduling information for the airplane’s propellers		N/A

Scheduling information for the airplane’s accessories		N/A

Scheduling information for the airplane’s instruments		N/A

Scheduling information for the airplane’s equipment		N/A

The degree of inspection for the airplane and subsystems	N/A

The applicable wear tolerances				N/A

Third-party documentation references			N/A

Recommended overhaul periods				N/A

Troubleshooting information				N/A

Storage limitations					N/A

Diagrams of structural access plates			N/A

Special inspection techniques				N/A

Protective treatments required				N/A

Data related to structural fasteners			N/A

List of special tools					N/A

Commuter category aircraft requirements			N/A