WEIGHT AND BALANCE  DOCUMENT

Ashby Glare Shield Products

Document WB-001

Revision A

November 19, 2005

Weight and Balance Document

Prior to being returned to service the aircraft Weight and Balance must be recalculated to reflect any changes in the Center of Gravity as a result of the glare shield installation.

The proper method for calculating a weight and balance may be found in FAA document FAA-H-8083-1 Aircraft Weight and Balance Handbook, Chapter 5.  This document may be obtained from the FAA.

The Ashby Glare Shields have the following nominal weights and CG locations.  Small variations can be expected in individual parts, and it would be prudent to weigh the specific part after trimming and before final installation.

| Part Number | Weight | DFE* |
| --- | --- | --- |
| PN6 | 3 lbs. 9 oz. | 6.75” |
| PN28 | 2 lbs. 15 oz. | 6.5” |
| PN 180 | 2 lbs. 4 oz. | 6.5” |
| PN 182 | 2 lbs. 9 oz. | 6.0” |
| PN172L | 2 lbs. 8 oz. | 6.375 |

“Distance from Edge” (DFE) is measured from the aft edge of the glare shield, at the center line, as shown in the following illustration:

C

L

Forward

DFE

Published Datum locations for applicable aircraft are as shown in figure 1.

Weight and Balance Document – Revision A  (cont.)

Figure 1 – Published Datum for applicable aircraft

| Glare Shield No. | Aircraft Series | Type Certificate Data Sheet | Datum |
| --- | --- | --- | --- |
| PN6 | PA32 | A3SO | 78.4” forward of leading wing edge. |
|  | PA34 | A7SO | 78.4” forward of leading wing edge from the inboard edge of the inboard fuel tank. |
| PN28 | PA28 | 2A13 | 78.4” forward of leading wing edge (straight wing only). 78.4” forward of inboard intersection of straight and tapered sections (semi-tapered wings). |
|  | PA44 | A19SO | 78.4” forward of leading wing edge at wing station 106. |
| PN172L | C172 – C172Q | 3A12 | Front face of firewall (28000 through 47746). Lower front face of firewall (17247747 through 17276654). |
|  | C172R,S | 3A12 | Lower portion of front face of firewall. |
|  | C172 | 3A17 | Lower front face of firewall. |
| PN180 | PA28 | 2A13 | 78.4” forward of leading wing edge (straight wing only). 78.4” forward of inboard intersection of straight and tapered sections (semi-tapered wings). |
| PN182 | C182 | 3A13 | Front Face of Firewall (older pre-1996 models). Lower portion of front face of firewall (1996 and newer models). |
|  | C180 | 5A6 | Front face of firewall. |
|  | C185 | 3A24 | Front face of firewall |