OPTIONAL LIGHTS INSTALLATION

12/24 volt (14/28)

Recommendation:  note the PMA number next to where the wires come out of the light strip, this should be entered into the aircraft  log book. Above the PMA number is the part number usually starting with the letter V Then several letters or numbers then a 12 or a 24 example VXXXXX12LEAEIOEL. That number is the voltage for that light strip.

Each 12-volt bulb pulls 60 milliamperes and each 24-volt bulb pulls 30 milliamperes. Count the bulbs and multiply accordingly.

A total of 10 12-volt bulbs will pull a maximum of 600

milliamperes, 10 24-volt bulbs will pull 300 milliamperes etc.

The power source you use should be a dimmable like the compass light or other instrument panel lights. Check and make sure the chosen circuit has the additional capacity. (Refer to 43-13-1a)

There is a small green 3-ampere fuse at the wire end of the light strip. However, I feel the lights should be wired to a protected circuit as well (fused or circuit-barker).

Attaching lights to the Glare shield lip.

I have found that clear silicone “general purpose glue & seal” works well. Hold the lights in place with clothes pins.

If you are in a hurry, a hot (fabric) glue gun works well.

Dennis Ashby